# Market-of-interest petrol and diesel outlook to 2030: model documentation

Prepared by Manish Ramjeawon for Nigel Zhuwaki's review, 14 October 2026. Draft: the changes from the analytical workbook v02 are proposals for your decision, not adopted inputs.

# 1. What the model does

It forecasts petrol and diesel demand from 2025 to 2030 for the market of interest (Gauteng, KwaZulu-Natal, Mpumalanga, Free State, North West, Limpopo) under Low, Base and High demand cases, and the volume that has to be brought into the area after Secunda and Natref output. It feeds pp16, 17, 19 and 20 of the South Africa market outlook pack (13102026_South_Africa_Market_Outlook_v1002).

| 2030, bn L | Low | Base | High |
| --- | --- | --- | --- |
| Petrol | 6.5 | 6.9 | 7.4 |
| Diesel | 11.2 | 11.9 | 12.8 |
| Total | 17.6 | 18.8 | 20.2 |
| Brought in (total less 6.3bn L Secunda + Natref) | 11.3 | 12.5 | 13.9 |

# 2. Files and how to run

| File | Role |
| --- | --- |
| s13_2025.py | Step 1-2: national 2025 demand (production + imports - exports) and the market-of-interest share from 2022 district sales; writes moi_2025.json (and builds pack slide 13). |
| moi_forecast.py | Steps 3-5: splits 2025 demand into uses, applies the drivers to 2030 for each case; writes moi_forecast.json. |
| moi_2025.json | 2025 starting point: national and market-of-interest petrol and diesel, Secunda + Natref output, district table. |
| moi_forecast.json | All assumptions used and results by case, year and use. |
| moi_forecast_results.csv | Same results as a flat table (case, year, petrol, diesel, total, brought in, each use). |

Run from the repository root (liquid_fuels_model), with this folder's path for the scripts:

python s13_2025.py PACK16.pptx OUT.pptx REPO_ROOT    then    python moi_forecast.py REPO_ROOT moi_forecast.json

Inputs come from the repository: assumptions/2026/reference/refinery_output_operators.csv; timeseries fuel_trade_sars.csv, fuel_prices_department.csv, vehicle_population_natis.csv, new_vehicle_registrations_natis_annual.csv; reference vehicle_parameters_stone2018.csv; data/raw/energy_dept/fsv_district/2022-Quarter4-Magisterial-Districts-data.xlsx; and output/delivered/sa_analytics_2026_10_11_v02/analytics.json (Segments, Plant supply and Evidence sheets).

# 3. Method, step by step

## Step 1. National demand in 2025

Demand = production + imports - exports, stock change nil. Production: Sasol's reported Secunda fuels output and Natref output (scaled from Sasol's 63.64% share) and Glencore's Astron output, converted to litres with the workbook's plant shares. Trade: SARS customs, calendar 2025. Result: petrol 9.29bn L, diesel 16.09bn L, total 25.4bn L (the workbook's sales-based 2025 figure is 21.0bn L; the gap is almost all diesel).

## Step 2. Market of interest in 2025

Each magisterial district's 2022 share of national Department sales, by product, is applied to 2025 national demand (190 districts in the six provinces). Shares: petrol 75.7%, diesel 69.5%. Result: petrol 7.0bn L, diesel 11.2bn L, total 18.2bn L. Secunda + Natref output (6.3bn L) is assumed to stay in the area.

## Step 3. Split into uses

The workbook's 2025 national segments (Segments sheet) are scaled to the market of interest by product share. Diesel power use is placed by location instead: Avon (670 MW, KwaZulu-Natal) is the only diesel peaker in the area; its 2025 use is independent-producer turbine output (FY2025, 662 GWh) x Avon's share of IPP capacity (670 of 1 005 MW) x 0.31 L/kWh. The rest of market-of-interest diesel is spread over the non-power uses in proportion. The workbook's 0.2bn L assumed private backup is not used (no published source); that diesel stays in the general uses. Demand above the workbook's sales-based segments is carried as 'unallocated'.

| 2025E use, bn L | Market of interest |
| --- | --- |
| Petrol: road | 6.50 |
| Petrol: other | 0.20 |
| Petrol: unallocated | 0.33 |
| Diesel: road freight | 3.55 |
| Diesel: other road (light vehicles, buses) | 1.18 |
| Diesel: mining | 1.03 |
| Diesel: agriculture | 0.83 |
| Diesel: manufacturing | 0.12 |
| Diesel: other | 1.41 |
| Diesel: unallocated | 2.91 |
| Diesel: Avon peaker | 0.14 |

## Step 4. Drivers applied to each use (year t = 2026 ... 2030, i = years since 2025)

GDP: index G(t) = product of (1 + g) over years; g flat at 1.0 / 2.0 / 3.0%.

Diesel activity factor = G(t) (diesel grows one-for-one with GDP). Petrol activity factor = product of (1 + 0.36 g) x price effect.

Price effect (petrol only): e(t) = e(t-1) + 0.405 x (-0.47 x ln P(t) - e(t-1)); multiplier = exp(e(t)). P(t) is the real price index vs 2025.

Efficiency: road petrol x (1 - e_p)^i; road freight and other road diesel x (1 - e_d)^i.

EVs: each year new light vehicles = 2025 eNaTIS new cars + LCVs x G(t); EV share of them from the S-curve; cumulative EVs / light fleet (fleet +1.95% a year) x 1.45 (new vehicles drive more) = share of light-vehicle fuel displaced. Applied to road petrol and other road diesel.

Rail: road freight diesel x (1 - shift x i/5), shift = (rail target - rail at its 2025 share in 2030) / road freight in 2030; total freight grows with Treasury's GDP path. Rail's own diesel is excluded.

Peakers: Avon diesel moves in a straight line from its 2025 level to 670 MW x load factor x 8.76 x 0.31 / 1 000 bn L in 2030.

Mining, agriculture, manufacturing, other and unallocated diesel grow with G(t) only; other and unallocated petrol with the petrol activity factor only.

## Step 5. Results

Petrol and diesel are the sums of their uses; brought in = total - 6.3bn L (Secunda + Natref, held at 2025E).

# 4. Assumptions and sources

| Assumption | Low | Base | High | Source |
| --- | --- | --- | --- | --- |
| GDP growth a year, 2026-30 | 1.0% | 2.0% | 3.0% | IMF WEO Oct 2026 (2026 forecast); Treasury 2026 Budget Review Table 2.2 (2028); Medium-Term Development Plan 2024-29 (3% target) |
| Petrol income response | 0.36 | 0.36 | 0.36 | Akinboade, Ziramba and Kumo (2008), Energy Economics; SA data 1978-2005 |
| Petrol price response (long run) | -0.47 | -0.47 | -0.47 | Same study |
| Share of price response felt per year | 40% | 40% | 40% | Brons et al. (2008) meta-analysis: short run -0.34, long run -0.84 |
| Real petrol price vs 2025, 2026 | +14.8% | +14.8% | +14.8% | Department petrol 95 inland: 2025 avg R21.66; 2026 Jan-Oct actual, Nov-Dec at Oct (R30.25); less 3.4% CPI |
| Real petrol price vs 2025, 2030 | +15% | +0% | -13% | Scenario choice: low holds 2026; base eases back; high mirrors low (AIA) |
| Efficiency a year, petrol / diesel | 0.8 / 0.7% | 0.65 / 0.55% | 0.5 / 0.4% | Pack p9: IEA/GFEI WP15 (2017) and IEA (2021) new-vehicle trend; eNaTIS turnover; Lightstone age; Stone et al. (2018) |
| EV S-curve (ceiling, midpoint, slope) | 90%, 2036, 2 | average of the two | 30%, 2040, 3 | AIA analysis (2025), vehicles.yaml |
| EV share of new light-vehicle sales, 2030 | 4.3% | 2.7% | 1.0% | Result of the S-curves |
| Rail freight in 2030 | 262m t | 250m t | 215m t | Draft National Rail Master Plan (viable rail market); Transnet target; 2025-26 growth of ~5% a year (Stats SA P7162) |
| Road freight moved to rail by 2030 | 7.2% | 6.1% | 2.8% | Result, against 2025 mode shares (Stats SA P7162) |
| Avon peaker load factor, 2030 | 1.4% | 4.5% | 10% | Eskom turbines 1 Apr-8 Oct 2026 (152 GWh); system operator MTSAO 2026-30 (below 6%); high: AIA |
| Diesel per kWh, peakers | 0.31 L | 0.31 L | 0.31 L | Eskom FY2024 and FY2025 fuel use / turbine output |
| Backup generators | none | none | none | Excluded: installed capacity not published (WITS gives unit counts only) |
| Rail's own diesel | none | none | none | Excluded at user's request; Transnet traction fuel implies ~17.5% (pack p7) |

# 5. Results by year

| Case | Fuel | 2025 | 2026 | 2027 | 2028 | 2029 | 2030 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Low | Petrol | 7.03 | 6.82 | 6.69 | 6.59 | 6.52 | 6.46 |
| Low | Diesel | 11.17 | 11.18 | 11.18 | 11.18 | 11.18 | 11.19 |
| Low | Total | 18.21 | 18.00 | 17.86 | 17.77 | 17.70 | 17.64 |
| Base | Petrol | 7.03 | 6.86 | 6.80 | 6.81 | 6.86 | 6.93 |
| Base | Diesel | 11.17 | 11.31 | 11.45 | 11.60 | 11.74 | 11.89 |
| Base | Total | 18.21 | 18.17 | 18.25 | 18.40 | 18.60 | 18.82 |
| High | Petrol | 7.03 | 6.89 | 6.91 | 7.02 | 7.20 | 7.43 |
| High | Diesel | 11.17 | 11.47 | 11.78 | 12.10 | 12.42 | 12.75 |
| High | Total | 18.21 | 18.36 | 18.69 | 19.11 | 19.62 | 20.18 |

# 6. What changed from the analytical workbook v02 (for your decision)

| Item | Workbook v02 | This model |
| --- | --- | --- |
| Area | National | Market of interest (six provinces) |
| 2025 starting point | 21.0bn L (sales-based) | 25.4bn L national (production + imports - exports); 18.2bn L in the area |
| Activity growth | 0.5 / 1.5 / 2.5% a year (analyst sensitivity) | GDP 1.0 / 2.0 / 3.0% (IMF / Treasury / MTDP); petrol at 0.36 x GDP |
| Petrol price | Not modelled | Elasticity -0.47, phased; scenario price paths |
| Efficiency | 1.5 / 1.0 / 0.5% a year (analyst sensitivity) | Petrol 0.8 / 0.65 / 0.5%; diesel 0.7 / 0.55 / 0.4% (SA fleet evidence) |
| EVs | Not in the scenarios | S-curve through the light-vehicle fleet |
| Rail | 15 / 7.5 / 2.5% of road freight (implies ~345 / 265m t) | 262 / 250 / 215m t by 2030 |
| Rail's own diesel | 10% rebound | Excluded |
| Power | 35 / 50 / 100% of 2025 utility + private power remaining | Avon peaker load factor 1.4 / 4.5 / 10%; backup generators excluded |

# 7. Limitations and points to review

Diesel 'unallocated' (2.9bn L in the area in 2025) is the gap between production + imports - exports and the workbook's sales-based segments. It grows with GDP. If part of 2025 is stock-building, 2025 and all cases are too high (the two-year average 2024-25 would be ~0.7bn L lower nationally).

District shares are fixed at 2022; Secunda and Natref output is held flat at 2025E and assumed to stay in the area.

Petrol elasticities come from 1978-2005 data; no newer SA estimate was found. Diesel has no price response.

Flat GDP of 2.0% in the base case is above Treasury's path for 2026-27 (1.6%, 1.8%); the rail calculation still uses Treasury's path.

EV S-curves give 0.2-0.4% of new sales in 2025 against 3.3% electrified sales in 2025 (mostly hybrids, which the efficiency rate covers); the curves may understate battery-EV take-up.

Avon's contract ends August/September 2030; the cases assume it runs through 2030.

The high-case price path and the 10% peaker load factor are scenario choices, not published figures.

Results are not yet in the repository's lfm model; the scripts read workbook v02 outputs and repository inputs directly.